EZ Flyer 230/135 Parallel Help for Windows 95

If the PP software does not load for Win'95 try these steps to resolve the Installation problem. First, turn off the system power and disconnect any printer which is connected on the pass through connector. Under Win'95, shutdown to reboot into DOS or boot up using F5 or F8 to go to the Safe Mode Command Prompt only option. This is being done to insure there are no drivers loading which can cause a driver or IRQ to load to cause an installation conflict. Put the SyQuest DOS/Win installation diskette 3 into the system for software versions less than 349. If all the software is on your cartridge and you only have the single install diskette, the drivers you will need to install the software are on the single install diskette. Put the diskette into A: or B:. Go to the A: or B: and type VISIT enter. Does visit load correctly? Remember the SQPPDRVR.SYS will ALWAYS make a drive letter even if the EPATSYQ.SYS installation is not successful. On software releases previous to 3.49, SQATDRVR.SYS /P was used as the driver in place of the SQPPDRVR.SYS. The SQATDRVR supported IDE drives as well as PP units if the /P option was specified on the command line. SQPPDRVR.SYS supports only SyQuest drives on the Parallel Port. EPATSYQ.SYS MUST load successfully. IF it does not load correctly, you must make sure the device is connected correctly and you can try this by itself from the command prompt. (c.1) A:> SQLOAD EPATSYQ.SYS /DE /DI /NI If this does not load correctly and say installation successful, you must go into the computer's CMOS setup and check the settings for the PP. If the settings are changeable, try selecting another option. Then after rebooting the system as per above, rerun the (c.1) command. If, this still does not work, try other PP options and retry again. If an option is not possible to select that works, either the PP cannot support our unit or the SyQuest PP unit is defective. You could try to install the SyQuest PP device on another PC system to see if there is a PP compatibility problem or the SyQuest PP is defective. You cannot go any further unless the EPATSYQ driver loads and successfully installs the drive. If the drive installs on another PC, the system may require a new PP adapter card. The default command to try all options would be to issue the below command with the options: (NI) do not initialize the PP control IC, (DE) disable checking for EPP mode, (DI) disable using interrupts, (R2) read speed nibble slow, (W2) write speed slow. (c.1) a:>SQLOAD EPATSYQ.SYS /NI /DE /DI /R2 /W2 NOTE: On some computers LPT1 setup cannot be changed, but LPT2's setup can be changed. Use LPT2 in these special systems and see if you can find a workable setup. If a PP setup correctly loads the EPATSYQ driver, the hardware is good. Now the system can be installed with known good hardware and installation conflicts can be isolated. (c.2) A:>SQLOAD SQPPDRVR.SYS or (c.2) A:>SQLOAD SQATDRVR.SYS /P This command will assign a drive letter to the SyQuest drive. The SQPPDRVR.SYS drivers uses the loaded EPATSYQ driver to assign a drive letter to the PP drive. If the driver does not load or the system hangs during this function, try adding these parameters to the SQATDRVR to prevent this drive from checking other IDE devices for a SyQuest drive. This is not necessary for the SQPPDRVR.SYS driver. A:>SQLOAD SQATDRVR.SYS /P -0 -1 -2 -3 The drive letter assigned to the SyQuest drive is taken from DOS. This may not be the same drive letter as you will normally be assigned. Other devices have not been loaded which may take this drive letter during normal installation. Do a DIR command for the EzFlyer PP drive letter. i.e. DIR D:. Does the directory command show a valid partition for the cartridge? If so, skip the next step of running SQATPREP. Next, if a new installation, the SQATPREP utility can be loaded to flip/format the cartridge to DOS mode. If you Run the following command: (c.3) A:> SQATPREP Using the SQATPREP utility, you must select the SyQuest drive and allow the prep to FLIP the cartridge from the MAC format to the PC format or to reformat a cartridge. NOTE: Do not run this PREP on the single install diskette version as all the install software on the cartridge under the SYQINST directory will be erased. This software also enables the format to select extended partitioning if desired. The automatic partitioning will format the cartridge with a primary partition. The automatic partitioning will erase all the free software so do not format the cartridge until you copied the free software of the cartridge if you desire. The cartridge must be in the PC format before the system can write/read to the cartridge. The SyQuest PREP utility must be used on the cartridge to flip the cartridge to PC format and erase all the MAC headers on track 0 if this cartridge is transported between the MAC and PC. Without removing the MAC headers, PC Exchange under the MAC system will be confused on seeing these MAC headers on the cartridge. DOS's FDISK and FORMAT does not write on track 0 to erase the previously stored data. Remember if you are not sure if the cartridge has been correctly partitioned (FDISKed), it is always advisable to run the SQATPREP utility to partition and format the cartridge instead of using the DOS's FORMAT command. Exit the PREP utility using the ESC key and you will be able to do copies and DIR to the drive letter assigned to the SyQuest drive by the SQPPDRVR driver. The hardware is now installed and operational. The Win'95 format command under My Computer will format the cartridge like a large floppy disk unless the cartridge has been flipped or had DOS's FDISK run on the cartridge to put a hard disk type partition first. This too may cause some problems when transporting the cartridge between operating systems. On DOS only cartridges (single installation diskette version software) this does not need to be done. The cartridge is prepared with a valid hard disk type partition. This debug procedure has bypassed all the drivers and software loading from the CONFIG.SYS, autoexec.bat and the normal Win'95 startup. The drivers/software may have conflicts with our device on the PP. If after loading the normal CONFIG.SYS and autoexec.bat during a reboot, the driver cannot be loaded successfully; there is a conflict with one of the installed drivers/devices. IRQ 5 and 7 are reserved by the system BIOS for the PPs. Some vendors cards, normally sound and network cards, ship their units set for IRQ 5 or 7 to lessen their exposure to conflicts. Since printer drivers generally do not use IRQs, there is no conflict. ECP printer setups do use IRQs if enabled in the systems CMOS. Since our PP device requires IRQs to operate fast and efficiently, there will now be a conflict. To check if this is the problem, open the Device Manager, and double click on Computer. This will bring up the View Resources and insure IRQ is selected. Read the IRQ lines labeled 05 and 07. One of these two must identify LPT1. If LPT1 does not appear on one of these two IRQ lines, then there is a device that has an IRQ assigned to the same IRQ as the printer's Parallel Port. The offending device using these reserved IRQs should be modified to use a different unused IRQ. The user must refer to the manual for this device or call the technical support line for this vendor's device to move it's IRQ. The EPATSYQ driver has modifiers. You can attach a "/DI" to the driver to disable IRQs during our operation. This will slow down the PP transfers. Under Win'95, through the control panel, system, device manager, SCSI controllers, SyQuest PP unit's settings, the Adapter settings window can be update to include the "/DI". It is highly recommended that the device using the reserved IRQ be changed to use one of their optional unused IRQs. Check for IRQ availability with a program like Device Manage. A "!" on the SyQuest Parallel Port device ICON is an indication that there is a conflict or the device is not attached properly. If an driver modifier was required under DOS to get the PP to recognize the drive, these same modifiers must be added to this Adapters settings window. If you have another Parallel Port Device, besides the printer, you might want to consider setting up an additional Parallel Port on your computer to accommodate both Parallel Port Devices. Parallel port devices cannot be chained together through the printer output connector on PP adapters. Once any Parallel Port IRQ conflicts have been adjusted, the SyQuest Parallel Port Drive should install in Win95. If you cannot reconfigure the IRQ conflict, you can use Win'95 settings to change the settings on the LPT port. Go into the Device Manager, select Ports ( Com & LPT), Printer Port (LPTx), Resources folder, and change the resource setting. The Resources input/output Range and Interrupt Request values can be changed to non conflicting values. Win'95 also provides some basic configurations which may be okay. If problems are still there, under the SCSI controllers, SyQuest Parallel Port Device, settings folder, and in the Adapter Settings window add the modifiers needed. The selected modifiers to the Adapter Setting Window below will tend to try to eliminate all conflicts. /ni /de /di /r2 /w2 /ni=no initialization is given to the PP control, /de=EPP check is disabled, /di=interrupts are not used, /r2=the read speed is nibble slow, and /w2=the write speed is slow. Where the port address is 140H and the IRQ is 10; and no PP initialization port=0x140 irq=10 /ni ADD Install95 If installing into Win'95 and following the instructions on diskette 2 for Win'95, the PP device failed to install. Click on START, select SETTINGS, go to CONTROL PANEL, Select ADD NEW HARDWARE and double click on it. When it asks if you wish to search for new hardware, select NO and then NEXT. It will give you a Hardware types window and select SCSI CONTROLLERS. The PP device uses a driver that resembles an ASPI driver to Win'95. The PP device will be shown under the SCSI CONTROLLERS device in the Win'95 Device Manager. Next insert diskette 2 into your floppy drive and select HAVE DISK. If the installation under DOS required modifiers to work properly, these modifiers may be added to the Win'95 driver also. Select the START, SETTINGS, CONTROL PANEL and this will bring you to the Device Manager window. Double click on SCSI CONTROLLERS, and SyQuest Parallel Port Device should be shown. Double click on SyQuest PP device, and open its SETTINGS folder. You can now enter the same modifiers to this Adapter Settings Windows. i.e." /DE /DI". Selecting the DRIVER folder, you can see the PP driver and its directory as: C:\Windows\System\Iosubsys\epathd.mpd If you brought up the Win'95 system without the cartridge installed or the external PP unit not powered up, you can install the unit without restarting Win'95. If Win'95 does not see the drive: 1. Go into the Control Panel, System, Device Manager and then double click on SCSI Controllers. 2. There should be a yellow "!" on the "SyQuest Parallel Port Device". a. Double click on the SyQuest parallel port device. b. Click "Driver" on the SyQuest PP device's Properties window. c. Click "Change Driver" button on the Driver's window. d. Click "OK" on Select Device Window. e. Click "OK" on Drivers Window will initialize the EPATHD.MPD driver. Now the driver is ready to use and the drive ICON should be available in the "MY COMPUTER" folder. Programs like Norton for DOS will not work on our drivers/cartridges if the system has put long file names and hidden files under Win'95. The DOS programs do not understand these files and will post an error. SCANDISK will not post an error, but will truncate the file name to its 8 characters name and 3 character extension for this long file name. Win'95 fully supports the DOS cartridges, but DOS programs will not understand the long file names developed by Win'95. DRIVE LETTER ID'S Under Win'95, PP units can change their drive letter assignments through the control panel's device manager. This can be used to move the SyQuest PP's assigned drive letter so the CDROM or other device can regain their original drive letter. DOS and Win'95 always assigns Winchester drives whether fixed or removable before CDROMs, tapes, etc. Select the SyQuest drive under the Device Manager's Disk drives ICON. Double clicking on this will bring up its settings folder. Go into the Setting folder and make sure Int13 and Removable are selected (an "X" in box). Go into the settings folder and select a Reserved drive letter to a higher UNUSED drive letter. DO NOT use Z:. When the system is rebooted to allow this change to take effect, the PP unit will now be assigned this drive letter and the other devices will move up a drive letter to fill in the vacated drive letter. You can now go back and redo this procedure to move the SyQuest PP drive ID to another unused letter. The printer pass through is for printers ONLY. Other PP adapters connected onto the Printer pass through will not function correctly. The PP adapters open lines to interrogate the data before passing it on to the pass through connector if it is not for the adapter's device. This will confuse the devices other than printers connected to the pass through connector. PP adapters can not be daisy chained. You must install multiple LPT (parallel ports) ports on your computer to use additional PP adapter units. Also care must be used when using A/B boxes with high speed parallel port units. A/B boxes are not recommended. The PP devices transfer data at higher rates than printers normally use. The additional cable length, resistance, and inductance of the switches can add noise on the cables. Noise on the cables will cause data corruption. The cables should be IEEE 1284 compliant which provide good noise immunity. If A/B switches are used, insure that you copy large files and compare these files as there is NO parity support on the printer ports to insure the data is transferred properly and not corrupted by cable noise. CDROMs can experience problems if the LASTDRV statement in config.sys is set to the CDROM address before the installation of our drive. This can cause statements such as invalid function when a DIRectory of the CDROM is requested. The solution is to increase the drive letter in the LASTDRV statement in the config.sys and then reboot the system. Win'95 normally does not need this statement. PRINTER DRIVER CONFLICTS The parallel port units use special reserved non printable characters to enable the hardware to know the next data being placed on the PP cable is for that device. Color printers and multiple font printers can also be using some of these special characters for their printer setups. This can cause conflicts between the two drivers and make each unit look defective to the system. A group of special characters has been set aside for PP devices that printer driver manufacturers will refrain from using. This has been accomplished after the horses have left the barn. Many printer companies like HP are in the process of rewriting drivers to stay clear of these newly reserved non printable characters. Some drivers still use these characters and will cause conflicts which cannot be resolved. The only way to correct this problem is to use one LPT port for the PP device and another LPT port for the printer. Some manufacturers have other drivers which will also drive the printer that uses characters that do not interfere with the PP devices. You should contact the printer manufacture to see if they have additional drivers that will not interfere with the PP device. Also, some companies like HP also have status drivers which monitor the printers status. If you have these printers connected on the pass through port of the controller, these drivers should be disabled. These drivers can also cause data corruption and system problems. HP has configuration files in the windows directories which can be run. The programs have options to remove the bilateral or bi-directional communication with the printer. Disabling the status communications does not affect the printing.